| PROJECT | FPU KELIDANG — PETRONAS CARIGALI BRUNEI LTD | DOC NO. | KLD-NVOE-MSE-SOW-3050SW-30001-A0 |
| --- | --- | --- | --- |
| TITLE | SCOPE OF WORK — SEA WATER COOLING PUMP & CENTRAL FW COOLING PUMP IMPELLER RENEWAL |  | REV: A0 |
| CONTRACT | PCBL/2023/PROJECT/003 | DATE | 08 May 2026 |

# 1.  PURPOSE AND SCOPE

This Scope of Work (SOW) defines the requirements for the supply, engineering, fabrication, testing, and delivery of replacement impellers for the Sea Water (S.W.) Cooling Pump and Central Fresh Water (CFW) Cooling Pump installed on the FPU Kelidang. The existing pumps are Naniwa Pump model FEV-250-2, originally certified by Lloyd's Register (LR), and are to be re-classed under American Bureau of Shipping (ABS) for the FPU conversion project.

# 2.  BACKGROUND

The FPU Kelidang is a ship-type Floating Production Unit permanently moored offshore Brunei Darussalam for the Kelidang Cluster Development Project (PCBL). The vessel is being converted from a COT vessel with a design life of 15 years and no scheduled drydocking (UWILD notation). As part of the conversion, impellers for the S.W. Cooling Pumps and Central CFW Cooling Pumps require renewal and re-certification under ABS.

# 3.  EQUIPMENT COVERED

| Tag No. | Equipment Description | Qty | Model | Service |
| --- | --- | --- | --- | --- |
| SW-308X01/02 | Sea Water Cooling Pump — Impeller Renewal | 2 sets | FEV-250-2 equiv. | Main S.W. Cooling |
| CW-308X01/02/03 | Central FW Cooling Pump — Impeller Renewal | 3 sets | FEV-250-2 equiv. | Central CFW Cooling |
| SW-308X01/02 | S.W. Pump — Casing Wear Rings | 2 sets | — | As required |
| All above | Mechanical Seals (all pumps) | 5 sets | — | As required |

# 4.  VENDOR SCOPE OF SUPPLY

## 4.1  S.W. Cooling Pump — Impeller

Supply of 2 new impellers (1 per pump) in AISI 316L Stainless Steel Cast, grade explicitly confirmed.

Original OEM drawing records only "Stainless Steel Cast" without grade — AISI 316L to be specified and certified.

Impellers to be dynamically balanced to ISO 1940-1 Grade G2.5 or better.

Full dimensional interchangeability with existing Naniwa FEV-250-2 casing to be confirmed with dimensional drawings.

ABS CLASS approval required.

## 4.2  Central CFW Cooling Pump — Impeller

Supply of 3 new impellers (1 per pump) in Cast Bronze ASTM B62 — same material as original OEM specification.

Material retained as original: Cu 87%, Sn 6%, Zn 6% — suitable for fresh water service at 55 °C over 15-year design life.

Impellers to be dynamically balanced to ISO 1940-1 Grade G2.5 or better.

Full dimensional interchangeability with existing Naniwa FEV-250-2 casing to be confirmed.

ABS CLASS approval required.

## 4.3  Casing Wear Rings (S.W. Pump)

Supply of new casing wear rings for S.W. Pumps (2 sets).

Material: Synthetic Resin or PEEK — compatible with seawater service; ABS-approved.

Proper clearance tolerances per OEM or equivalent specification.

## 4.4  Mechanical Seals

Supply of new mechanical seals for all 5 pumps.

S.W. Pump seals (2 sets): SiC vs SiC faces; EPDM or FKM elastomers.

CFW Pump seals (3 sets): Carbon vs Ceramic faces; EPDM elastomers.

## 4.5  Associated Hardware

Impeller nuts, keys, plate washers, spring washers — SS316L for S.W. pumps; Copper Alloy / SS304 for CFW pumps.

O-rings (EPDM or NBR as applicable).

All fasteners: A4-70 SS316L.

## 4.6  Documentation

Material Test Certificates (EN 10204 Type 3.1) for all pressure-wetted parts.

Dimensional drawings confirming interchangeability with existing pump casings.

Dynamic balance test reports (ISO 1940-1).

Installation and Maintenance Manual.

ABS CLASS certificate or letter of approval.

# 5.  PURCHASER SCOPE (FOR INFORMATION)

Pump disassembly, inspection, and reassembly.

Installation of new impellers and associated parts by CONTRACTOR mechanical team.

Alignment of pump and motor post-reassembly.

Performance verification after installation.

Disposal of replaced worn parts.

# 6.  EXCLUSIONS

Pump casings, motors, bearings, shafts, and couplings (unless identified during inspection).

M/E Jacket FW Cooling Pump (not in scope).

Civil / structural works.

Electrical cable works beyond pump terminal box.

# 7.  KEY MILESTONES

| Milestone | Responsibility | Target |
| --- | --- | --- |
| Purchase Order Issued | COMPANY | TBD |
| Vendor Drawing Submittal | VENDOR | +4 weeks from PO |
| Drawing Approval | COMPANY / ABS | +2 weeks from submittal |
| Delivery to Site | VENDOR | TBD |

# 8.  STANDARDS AND CODES

ABS Rules for Building and Classing Floating Production Installations

ISO 1940-1: Balance quality requirements for rotors in constant (rigid) state

ASTM B62: Standard Specification for Composition Bronze or Ounce Metal Castings

ASTM A743/A743M: Castings, Iron-Chromium, Iron-Chromium-Nickel, Corrosion Resistant

EN 10204: Metallic products — Types of inspection documents

ASME B31.1: Power Piping (applicable to pump connections)

# 9.  COMPANION DOCUMENTS

KLD-NVOE-MSE-SPN-3050SW-30001 — Equipment Specification

KLD-NVOE-MSE-TDS-3050SW-30001 — Technical Datasheet

KLD-NVOE-MSE-TRP-3050SW-30001 — Technical Requisition Package