| PROJECT | FPU KELIDANG — PETRONAS CARIGALI BRUNEI LTD | DOC NO. | KLD-NVOE-MSE-SPN-3050SW-30001-A0 |
| --- | --- | --- | --- |
| TITLE | EQUIPMENT SPECIFICATION — SEA WATER COOLING PUMP & CENTRAL FW COOLING PUMP IMPELLER |  | REV: A0 |
| CONTRACT | PCBL/2023/PROJECT/003 | DATE | 08 May 2026 |

# 1.  GENERAL

This specification defines the minimum technical requirements for the supply of replacement impellers, wear rings, and mechanical seals for the Sea Water (S.W.) Cooling Pump and Central Fresh Water (CFW) Cooling Pump on FPU Kelidang. The FPU has a 15-year design life with no scheduled drydocking (UWILD notation). All equipment shall be ABS CLASS approved.

# 2.  DESIGN BASIS

| Parameter | Value | Remarks |
| --- | --- | --- |
| Project | FPU Kelidang — Kelidang Cluster Development Project |  |
| COMPANY | PETRONAS Carigali Brunei Ltd (PCBL) |  |
| Classification | American Bureau of Shipping (ABS) | Full CLASS approval required |
| Original CLASS | Lloyd's Register (LR) | Re-class to ABS |
| Design Life | 15 years | No drydocking; UWILD notation |
| Location | Offshore Brunei Darussalam | Tropical marine |
| Ambient Temp. | 20–38 °C |  |
| Relative Humidity | Up to 100 % | Saline marine atmosphere |
| Design Codes | ABS FPI Rules; ISO 1940-1; ASME B31.1 |  |
| Reference P&ID | KLD-NVOE-TSA-PID-3050SW-30001-001 | Rev. B0 |

# 3.  SEA WATER COOLING PUMP — IMPELLER SPECIFICATION

## 3.1  Pump Service Data

| Service | Main Seawater Cooling |  |
| --- | --- | --- |
| Tag No. | SW-308X01 / SW-308X02 |  |
| Quantity | 2 sets (1 Duty + 1 Standby) |  |
| Pump Type | Vertical centrifugal, single stage |  |
| Original Model | Naniwa FEV-250-2 (LR certified) |  |
| Rated Flow | 420 m³/h | Minimum at rated speed |
| Total Head | 21 m |  |
| Suction Head | –5 m | Below pump centreline |
| Pump Speed | 1,750 RPM | 4-pole, 60 Hz |
| Hydrostatic Test Press. | 0.6 MPa (G) | 30 min minimum |
| Motor | 37 kW, 440 V, 3Ph, 60 Hz, IP56 min. |  |
| Suction / Discharge | DN250, JIS 5K flanged |  |
| Operating Fluid | Seawater |  |
| Classification | ABS (original: LR) |  |

## 3.2  Impeller Technical Requirements

The original OEM drawing records the impeller material as "Stainless Steel Cast" without specifying a grade. For ABS re-classification and 15-year no-drydock service in seawater, the grade shall be explicitly confirmed as AISI 316L minimum.

| Material | AISI 316L Stainless Steel Cast | Grade explicitly specified — OEM grade was unconfirmed |
| --- | --- | --- |
| Reason for Spec. | Seawater service (Cl⁻ environment); 15-yr no-drydock; ABS re-class | 316L Mo content provides crevice/pitting corrosion resistance |
| Dynamic Balance | ISO 1940-1 Grade G2.5 or better | Both faces; certificate required |
| Dimensional Tolerance | Full interchangeability with FEV-250-2 casing | Dimensional drawing required with bid |
| Trimming | Trim to rated duty point | Hydraulic calculation to be provided |
| NDT | Dye penetrant (DP) test — 100% cast surfaces | ABS witnessed; report required |
| CLASS Certificate | ABS material + product certificate | Required before dispatch |
| Hardware | Nut / Key / Washers — SS316L | 1 set per impeller |

## 3.3  Casing Wear Rings

| Material | Synthetic Resin or PEEK | Seawater compatible; ABS approved |
| --- | --- | --- |
| Clearance | Per OEM (FEV-250-2) tolerance | STC |
| Quantity | 2 per pump — casing ring + impeller ring | 4 rings total (2 pumps) |

## 3.4  Mechanical Seal

| Type | Single mechanical seal, external |  |
| --- | --- | --- |
| Seal Face Material | Silicon Carbide (SiC) vs SiC | Seawater service |
| Elastomer | EPDM or FKM | Seawater compatible |
| Pressure Rating | ≥ 0.6 MPa (G) |  |
| Temperature | ≤ 60 °C |  |
| Quantity | 1 set per pump — 2 sets total |  |

# 4.  CENTRAL FW COOLING PUMP — IMPELLER SPECIFICATION

## 4.1  Pump Service Data

| Service | Central Fresh Water Cooling |  |
| --- | --- | --- |
| Tag No. | CW-308X01 / 02 / 03 |  |
| Quantity | 3 sets (2 Duty + 1 Standby) |  |
| Pump Type | Vertical centrifugal, single stage |  |
| Original Model | Naniwa FEV-250-2 (LR certified) |  |
| Rated Flow | 360 m³/h | Minimum at rated speed |
| Total Head | 25 m |  |
| Suction Head | +15 m |  |
| Pump Speed | 1,750 RPM | 4-pole, 60 Hz |
| Operating Temp. | 55 °C max | Fresh water service |
| Hydrostatic Test Press. | 0.8 MPa (G) | 30 min minimum |
| Motor | 37 kW, 440 V, 3Ph, 60 Hz, IP56 min. |  |
| Suction / Discharge | DN250, JIS 5K flanged |  |
| Operating Fluid | Fresh Water (cooled) |  |
| Classification | ABS (original: LR) |  |

## 4.2  Impeller Technical Requirements

Original material Cast Bronze (ASTM B62) is retained. At 55 °C fresh water service with a 15-year design life, Cast Bronze ASTM B62 is a proven and industry-accepted material. No material upgrade is required.

| Material | Cast Bronze — ASTM B62 | Same as OEM original |
| --- | --- | --- |
| Composition | Cu: 87%, Sn: 6%, Zn: 6% (max) | Per ASTM B62 |
| Rationale | 55 °C FW service; 15-yr design life; proven in-service record | No dezincification risk at 55 °C FW; upgrade not warranted |
| Dynamic Balance | ISO 1940-1 Grade G2.5 or better | Both faces; certificate required |
| Dimensional Tolerance | Full interchangeability with FEV-250-2 casing | Dimensional drawing required |
| NDT | Dye penetrant (DP) test — 100% cast surfaces | ABS witnessed; report required |
| CLASS Certificate | ABS material + product certificate | Required before dispatch |
| Hardware | Impeller Nut: Copper Alloy Rod (BS2874) | Key / Washers: SS304 — per OEM |

## 4.3  Mechanical Seal

| Type | Single mechanical seal, external |  |
| --- | --- | --- |
| Seal Face Material | Carbon vs Ceramic | FW service at 55 °C |
| Elastomer | EPDM O-rings | FW compatible |
| Pressure Rating | ≥ 0.8 MPa (G) |  |
| Temperature | ≤ 70 °C |  |
| Quantity | 1 set per pump — 3 sets total |  |

# 5.  GENERAL REQUIREMENTS — ALL PUMPS

| Interchangeability | New impellers shall be fully interchangeable with existing pump casings | Dimensional drawings required with bid |
| --- | --- | --- |
| Spare Parts | 2-year operational spares list with bid |  |
| Marking | Material grade, heat number, part number permanently marked on each impeller |  |
| Packaging | Export-grade; corrosion-inhibited; suitable for sea freight |  |
| Warranty | Minimum 24 months from date of installation |  |
| Documentation | IOM Manual, Material Certs (3.1), Balance Reports, CLASS Certificate, Drawings | All in English |